22.02.2021

STARTER-FP V1

OPERATING MANUAL

The STARTER-FP controller controls the metering roller's revolutions (rpm) in the seed spreader, maintaining the metering roller's rpm set by the user. The rpm (revolutions per minute) are displayed on the device screen. Previously the user shall read the rpm value in the charts for the required sowing dose [kg/ha], assumed travel speed [km/h] and the machine's working width [m].

The charts quote the weight of seeds sown in 1 minute M[kg/min].

For the required dose D[kg/ha], the weight should be:

M[kg/min] = ( D[kg/ha] * S[m] * V[km/h] ) / 600

where:

V[km/h] is the travel speed during the procedure

S[m] is the machine's working width

The device does not measure the travel speed, so the user has to decide at what speed they carry out the procedure and then control it with the speedometer embedded in the tractor.

Once you calculate the required bulk weight M[kg/min] based on the formula above, read the metering roller's corresponding rpm in the chart. Then, enter the rpm into the device's memory (item 4 of the Manual).

Due to the possible discrepancy between the actual result and the one given in the chart, it is recommended to carry out a sowing test (item 5 of the Manual). During the sowing test, the metering roller rotates at the entered rpm for 1 minute. After weighing the fertiliser spread, check if the weighed weight is the same as the calculated value M[kg/min].

The device also activates the blower's motor.

Connect the controller to the seat mounted on the box. A point is displayed in the bottom corner of the screen after 12 V is supplied to the box. The controller is in standby mode.

If the power supply is connected the other way round, the device will not be damaged.

- CONTROLLER ON/OFF

Activate the controller by pushing the  button once.

Fig. 1

Power supply on/off

The name of the device and the current software version is displayed, as in Fig. 2.

Fig.  2.  Start screen

After releasing the  button, the operation screen is displayed.

In order to switch off the controller, press and hold the   button until a dot appears on the display.

- WORK CYCLE

Figure 3 shows the work screen view. The metering roller's pre-programmed rpm is shown in negative on the right. The icons of the metering roller (left - top) and the blower (left - bottom) are not flashing, which means that neither of them is rotating.

Fig. 3

Work screen The fan and metering roller off

In order to start sowing, press the  button. The screen changes, as shown in Fig. 4.

Fig. 4

Operation start. Activating the blower motor

The fan icon starts flashing, indicating that the fan motor is on. The time left to activate the metering roller is displayed simultaneously.

After the time countdown is finished (time setting - see item 8 of the Manual), the metering roller is activated, signalled with the metering roller icon flashing. The actual rpm of the metering roller is displayed, as in Fig. 5.

Fig. 5

Operation start. Metering roller on

When you approach a headland, press the  button. The metering roller stops, but the fan continues working. An image is displayed, as shown in Fig. 6.

Fig. 6

Sowing break

If you want to continue working, press the  button again. The metering roller is on again.

If you want to stop working, press and hold the  button for 1 second. The fan switches off, and after the fan switch off time is counted down, an image is displayed, as shown in Figure 3.

If anomalies occur during work, they are displayed on the screen. Displaying the anomalies is accompanied by an intermittent buzzer sound. The anomalies are described in Section 10.

3. TANK EMPTYING

When the controller is on, and the image shown in Fig. 3 is displayed on the screen, press and hold the  button for about 1 second. When you hear the buzzer's double sound and a new image is displayed, release the  button.

Fig.  7

Tank emptying

In order to start emptying the tank, press the  button. The image starts flashing. Sowing of the seeds from the metering unit starts at maximum speed. The fan is not working. In order to stop emptying the tank, press the  button again.

After the tank emptying is completed, press the  button and return to the Work Screen. Press and hold the button to switch off the controller.

4. SETTING THE METERING ROLLER'S RPM

In order for the controller to maintain the correct sowing dose, enter the metering roller's required rpm. The rpm calculation method is described on page 1.

In order to set the metering roller's required rpm when the controller is on, and the image shown in Fig.  3 is displayed on the screen, press and hold the  button for about 1 second.

The buzzer's double sound is emitted, and Figure 7 is displayed.

Release the  button.

Press and hold the  button until the image is displayed, as in Fig. 8.

Fig.  8

Setting the metering roller's rpm

Press the  button. The "rpm" pictograms on the screen start flashing.

Use the  button to increase the metering roller's required rpm up to 70.

Use the  button to decrease the metering roller's required rpm to min. 5.

After setting the correct number of revolutions, press the  button and return to the Work Screen. Press and hold the button to switch off the controller.

5. SOWING TEST

In order to compare the actual seed sowing in 1 minute with the value read from the chart, you can carry out a sowing test.

To that end, when the controller is on, and the image as in Fig. 3 is displayed on the screen, press and hold the  button for about 1 second. The buzzer's double sound is emitted, and Figure 7 is displayed.

Release the  button.

Press and hold the  button until the image is displayed, as in Fig. 9

Fig.  9

Sowing test

In order to activate the metering roller's rotation, press the  button. The second counter starts counting down to 0, and the image starts flashing. Sowing of the seeds from the metering unit starts at the pre-set value of the metering roller's rpm. The fan is not working. After 60 seconds, the metering roller stops, and the image stops flashing.

You can stop the metering roller's rotation any time by pressing the  button and start the test again.

After the sowing test, press and hold the  button and move on to setting the metering roller's rpm. Press and hold the button to switch off the controller.

After weighing the sown seeds, you can check if they correspond to the required dose and adjust the metering roller's rpm if necessary.

6. BLOWING THROUGH

When the controller is on, and the image shown in Fig. 3 is displayed on the screen, press and hold the  button for about 1 second. The buzzer's double sound is emitted, and Figure 7 is displayed.

Release the  button.

Press and hold the  button until the image is displayed, as in Fig. 10

Fig.  10

Blowing the system through

In order to start blowing through, press the  button. The image starts flashing. The fan starts working. The metering roller is not working. In order to stop blowing through, press the  button again.

After blowing the system through, press the  button and return to the Work Screen. Press and hold the button to switch off the controller.

7. ADJUSTING THE DISPLAY BRIGHTNESS

In order to improve the working comfort, you can set any of the display 6 brightness grades.

To that end, when the controller is on, and the image as in Fig. 3 is displayed on the screen, press and hold the  button for about 1 second. The buzzer's double sound is emitted, and Figure 7 is displayed.

Release the  button.

Press and hold the  button until the image is displayed, as in Fig. 11

Fig.  11

Setting the display brightness

Press the  button. The sun pictogram starts flashing on the screen.

Use the  button to increase the display brightness.

Use the  button to decrease the display brightness.

After setting the correct brightness, press the  button and return to the Work Screen. Press and hold the button to switch off the controller.

8. SETTING THE NUMBER OF THE METERING ROLLER PULSES PER REVOLUTION

In order to set the number of pulses per one revolution of the metering roller when the controller is on, and the image shown in Fig.  3 is displayed on the screen, press and hold the  button for about 1 second. The buzzer's double sound is emitted, and Figure 7 is displayed.

Release the  button.

Press and hold the  button until the image is displayed, as in Fig. 12

Fig.  12

Setting the number of the metering roller pulses per revolution

Press the  button. The pictograms on the screen start flashing.

Use the  button to increase the number of the metering roller pulses up to 99.

Use the  button to decrease the number of the metering roller pulses.

After setting the correct number of pulses, press the  button and return to the Work Screen. Press and hold the button to switch off the controller.

9. SETTING THE DELAY BETWEEN THE FAN AND METERING ROLLER OPERATION START/STOP

Set the delay between the fan and the metering roller's motor starting at the beginning of work. It also indicates the fan operation time after the metering roller stops at the end of work.

In order to set the delay when the controller is on, and the image shown in Fig.  3 is displayed on the screen, press and hold the  button for about 1 second. The buzzer's double sound is emitted, and Figure 7 is displayed.

Release the  button.

Press and hold the  button until the image is displayed, as in Fig. 13

Fig.  13

Setting the delay between the fan and the metering roller operation start

Press the  button. The pictograms on the screen start flashing.

Use the  button to increase the time up to 9 seconds.

Use the  button to decrease the time to min. 1 second.

After setting the correct delay setting, press the  button and return to the Work Screen. Press and hold the button to switch off the controller.

10. EMERGENCIES

The supply voltage and the seed rotation motor's limiting current are controlled during work.

LOW SUPPLY VOLTAGE

If the supply voltage value drops below 9 V, a flashing icon appears on the screen, as shown in Fig. 14.

Fig.  14

Supply voltage too low

HIGH SUPPLY VOLTAGE

If the supply voltage exceeds 18 V, a flashing icon appears on the screen, as shown in Fig. 15.

Fig.  15

Supply voltage too high

SHORT-CIRCUIT IN THE METERING ROLLER'S MOTOR

If a short circuit is discovered in the metering roller's motor power supply circuit, a flashing image is displayed, as shown in Fig. 16. Check what causes the short circuit in the valve - the metering roller's motor or the cable supplying voltage to the motor.

Fig.  16

Short-circuit in the metering roller's motor circuit

NO PULSES DURING THE METERING ROLLER'S MOTOR START

If 3 sensor pulses are not detected during the metering roller's motor start, the flashing image shown in Figure 17 is displayed. During the motor start, if no sensor pulses are supplied, the voltage increases gradually to reach the maximum supply voltage. Check the reason for the lack of pulses from the sensor - the metering roller's motor does not rotate, or the sensor does not capture the pulses from the metal cam.

Cancel the message with the  button.

Fig.  17

The metering roller's motor does not start